РУКОВОДСТВО ПО СБОРКЕ СТАНЦИЙ

Station assembly guide

| Конфигурация / Configuration | EVT-PRE-20 |
| --- | --- |
| Ревизия / Revision | REV_D7 |
| Дата / Date | 16.10.2026 |
| Статус / Status | ГОТОВ К ПЕРЕДАЧЕ / READY FOR HANDOFF |
| Объём / Scope | 1 управляемый лот, 20 станций / one controlled lot, 20 stations |

1. Входной контроль / Incoming inspection

RU: До сборки сверить ревизию, MPN, количество, упаковку, ESD/MSL, механические размеры, сертификаты и результаты фабричного контроля. Несоответствие изолировать с NCR.
EN: Before assembly verify revision, MPN, quantity, packaging, ESD/MSL, mechanical dimensions, certificates and factory inspection results. Segregate nonconforming material under NCR.

2. Последовательность / Sequence

| Шаг | Операция / Operation | Критерий / Acceptance |
| --- | --- | --- |
| MFG-010 | Verify released manufacturing bundle | All hashes match and no OPEN blocker authorized for build |
| MFG-020 | Verify materials and deviations | All items accepted or deviation signed |
| MFG-030 | Select lot and assign serials travellers and housing plan | DIO-EVT-001 through DIO-EVT-020 assigned with no duplicates; all primary housings vacuum cast; 3D fallback controlled |
| MFG-040 | Stencil paste placement and reflow | Profile within frozen paste limits and traceable batch |
| MFG-050 | AOI visual and X-ray | IC-004 and IC-005 accepted |
| MFG-060 | Bare power resistance check | No rail-to-ground short against released limits |
| MFG-070 | First power with current limit | No latch-up overheating or current limit event |
| MFG-080 | Program immutable boot foundation | Readback version and protection state match release |
| MFG-090 | Program station and coprocessor test builds | Hashes and versions match selected release |
| MFG-100 | PCB_MAIN boundary scan and interface self-test | All fitted interfaces including both SIM slots enumerate and test points pass |
| MFG-110 | PCB_PWR load and protection test | All rails protection and temperature within frozen limits |
| MFG-120 | PCB_MIC individual acoustic test | Identity sensitivity noise and phase within frozen limits |
| MFG-130 | Build and verify harnesses | 100 percent continuity polarity labels and length pass |
| MFG-140 | Install PCB and antennas in electronics module | No interference; antennas and coax routing match drawing |
| MFG-150 | Assemble four-microphone geometry | Triangle sides and MIC4 offset within released tolerance |
| MFG-160 | Install seals membranes and vents | Correct lot orientation compression and clean acoustic ports |

3. Критические правила / Critical rules

• Серийный номер и traveller назначаются до начала сборки. / Assign serial number and traveller before assembly starts.

• RF порты не включать без согласованной нагрузки/антенны. / Do not transmit into an unloaded RF port.

• Соблюдать полярность батареи, предохранитель у батареи и запрет заряда ниже 0 °C. / Observe battery polarity, battery-side fuse and the below-zero charging prohibition.

• Четыре MIC-жгута одинаковой длины; лишнюю длину укладывать сервисной петлёй. / All four MIC harnesses have equal length; place surplus in a service loop.

• После любого ремонта повторить затронутый тест и полный регрессионный EOL. / After rework repeat the affected test and the full regression EOL.

4. Запись результатов / Records

RU: Для каждого шага фиксируются исполнитель, дата, оборудование, версия процедуры, результат, ссылка на фото/измерения и хеш evidence. Пустое обязательное поле блокирует PASS.
EN: For each step record operator, date, equipment, procedure revision, result, photo/measurement reference and evidence hash. A blank mandatory field blocks PASS.